Human Activity Recognition (HAR) on Edge Hardware

An AIoT Prototype Report

# 1. Prototype

## 1.1 Problem and objectives

Human activity recognition lets a wearable or fixed node know whether its user is resting, walking or has fallen, without a camera. The conventional approach streams raw inertial data to the cloud. For six 16-bit channels at 100 Hz this is 1.2 kB/s (about 4.3 MB per hour) per device. It keeps the radio, the most expensive consumer on a battery-powered node, permanently on; adds round-trip delay to a time-critical event such as a fall; exposes behavioural data to third parties; and stops working when connectivity is lost.

This project therefore moves the intelligence to the sensor. An ESP32 reads an MPU6050 inertial sensor, classifies each 1.28 s window locally as Stationary, Walking or Fall Event, and uses the radio only to report an event (one label instead of 1.2 kB/s). Following the prototyping framework of the course, the goal is stated as a falsifiable hypothesis with an operating condition, an observable metric and a pass/fail threshold:

- H1. With the node worn at the waist and running on battery, the INT8 model on the ESP32 classifies the three activities with ≥ 90% fall recall, completes inference in ≤ 50 ms per window, fits in ≤ 100 KB of flash and ≤ 16 KB of tensor arena, and needs no cloud connection. In this report H1 is checked on size and memory (computed) and on latency (estimated); fall recall still has to be validated on real data (Section 4.1).

## 1.2 Evaluation in the context of the course (Chapter 1.6)

Judged against Chapter 1.6, the prototype is an AIoT system rather than a stand-alone IoT or machine-learning demo, because all three layers are present and each constrains the others: the sensor rate fixes the window and the model input size; the model size and latency decide which microcontroller can host it; and the inference time sets the CPU duty cycle and hence the energy budget. In terms of fidelity, it is an early, design-level prototype: the sensing and windowing pipeline is specified, and a reference model has been implemented, quantised and measured for size and memory, but only on synthetic data and not yet on a physical ESP32 + MPU6050. It therefore tests the feasibility part of H1 (does it fit and run within the budget) and not yet the real-world part (does it recognise real people and falls). Section 4 lists what remains.

How it addresses the problem. Local inference removes the continuous uplink (bandwidth and energy), removes network round-trip from the decision (latency), keeps raw movement data on the device (privacy), and lets the node keep working during outages (robustness).

## 1.3 Development process, design decisions and rejected alternatives

- Idea and low fidelity. Storyboards and workflow diagrams traced how movements appear in inertial signals and exposed edge cases early, for example that dropping into a chair resembles a fall. This led to the decision to keep the problem to three classes.

- Reference implementation. A 1D-CNN was written and trained in TensorFlow/Keras on simulated IMU windows, then converted to TensorFlow Lite in FP32 and full-integer INT8 form. This gave real figures for model size, activation memory and operation count (Section 3). Hardware wiring and the 100 Hz timer were designed to be validated next in the Wokwi simulator, then on the board.

- Optimisation. Quantisation was applied and compared against FP32 (Section 3).

Alternatives were considered and rejected at the design stage, on analysis rather than on experiment:

- Cloud classification of streamed raw data: 1.2 kB/s per device keeps the radio on, adds network delay to fall alerts, and fails offline. This is the problem the project sets out to avoid.

- A threshold rule on acceleration magnitude: it is tiny, but cannot separate a fall from sitting down abruptly or from stumbling, and thresholds do not transfer between people.

- LSTM or transformer classifiers: they carry state or memory that grows with window length and are harder to quantise, which is a poor fit for an ESP32.

- FIFO + data-ready interrupt reading: better for energy, but harder to debug; kept as future work.

Two design decisions shaped the rest of the system: timer-driven polling instead of interrupts, for deterministic timestamps, and 128-sample windows with 50% overlap, so that an event falling across a window boundary is still seen whole in the next window.

# 2. Sensing Pipeline

The pipeline turns physical motion into a class label in six stages (Figure 1). Each choice below is justified by the problem: detect walking and falls within about a second, on a battery, without the cloud.

| MPU6050 6 axes | I2C 100 kHz 14-byte burst | 100 Hz timer ring buffer | Window 128×6 50% overlap, normalise | INT8 1D-CNN (on ESP32) | Label/fall alert (event only) |
| --- | --- | --- | --- | --- | --- |

Figure 1. Sensing and inference pipeline. Only the final stage may use the radio.

## 2.1 Hardware, sensors and communications

| Aspect | Choice | Justification |
| --- | --- | --- |
| Microcontroller | ESP32 (dual-core, 240 MHz, 520 KB SRAM) | Enough SRAM for the tensor arena and window buffers; one core can sample while the other infers; built-in BLE/Wi-Fi for event alerts; low cost. |
| Sensor | MPU6050: 3-axis accelerometer + 3-axis gyroscope, 16-bit ADC | Gyroscope adds rotation information that helps separate a fall from sitting down; compact, cheap, motion-detect interrupt for wake-up. |
| Signals and range | AX, AY, AZ at ±2 g; GX, GY, GZ at ±250°/s | Highest resolution (16384 LSB/g) for gentle motion such as walking and posture. The limit for fall impacts is discussed in Section 4.1. |
| Reading mode | Timer-driven polling, one 14-byte burst read per tick | Fixed 10 ms period gives deterministic timestamps. A burst of 17 bytes at 100 kHz takes about 1.6 ms, only 16% of the period. FIFO + data-ready interrupt was considered but is more complex to debug. |
| Sampling and window | 100 Hz; 128 samples (1.28 s), 50% overlap, new window every 0.64 s | Body motion lies mostly below ~20 Hz, so 100 Hz leaves ample margin and still resolves impact transients. 1.28 s covers at least one gait cycle; 50% overlap halves the chance of splitting an event. |
| Bus | I2C, 100 kHz, 4.7 kΩ pull-ups on SDA/SCL | Robust over jumper wires; 400 kHz is available if throughput is ever needed. |
| Communication | Radio off during sampling and inference; used only to send a fall alert (BLE) | Transmission costs roughly an order of magnitude more current than computation, so it is event-based rather than streaming. |

## 2.2 Energy budget (estimate)

No current measurements exist yet, so the budget is an estimate from datasheet-level values: ESP32 CPU active without radio about 40 mA (datasheet range 30–68 mA), light sleep about 0.8 mA, Wi-Fi/BLE transmit above 100 mA, MPU6050 about 3.9 mA. Assumptions: the CPU is awake 2 ms of every 10 ms sampling tick (I2C burst plus buffering) and sleeps otherwise; inference takes up to 17 ms every 640 ms (Section 3.3); the radio is used only for rare alerts. Then I_avg ≈ 8.6 mA (sampling) + 1.0 mA (inference) + 3.9 mA (sensor) ≈ 14 mA, or roughly 35 hours from a 500 mAh cell, before regulator losses. Two conclusions follow. First, the sampling loop and the sensor dominate; inference is only about 7% of the total, so putting the sensor into its low-power motion-detect mode while the user is stationary is the most valuable energy saving. Second, streaming the data would be far worse, since a radio at more than 100 mA would multiply the average current several times. Development boards also draw extra current from their USB chip, so a bare module is needed to approach this figure. The estimate must be replaced by a measurement in future work.

# 3. On-Device AI

## 3.1 Model and training data

The classifier is a compact 1D convolutional neural network: input 128×6 (normalised per channel with training-set mean and standard deviation) → Conv1D(16 filters, kernel 5, ReLU) → MaxPool(2) → Conv1D(32, kernel 5, ReLU) → GlobalAveragePooling → Dense(3, softmax). It has 3,187 parameters and about 208,000 multiply–accumulate operations (MACs) per window. A 1D-CNN was chosen because it learns local temporal patterns with shared weights (few parameters) and uses only operators with efficient INT8 kernels. Recurrent networks keep state and are harder to quantise; transformers need memory that grows with the square of the window length.

Data. No real recordings were available, so the model was trained on synthetic windows: 24 simulated subjects (18 for training, 6 for testing, so the test subjects are unseen), 100 windows per class and subject (5,400 training and 1,800 test windows). The generator varies body orientation, gait frequency and amplitude, and noise per subject. Falls contain a free-fall phase, an impact spike of 3–6.5 g and a new resting orientation; walking and stationary windows follow the same sensor model. Synthetic data is only suitable for checking that the pipeline, memory and quantisation work; it cannot show how well the system recognises real people (Section 4.1). The generator and training script are provided with the report so that every figure can be reproduced.

## 3.2 Optimisation: post-training INT8 quantisation

Quantisation was selected because it targets exactly the three constraints of the device (flash, compute and SRAM) and the ESP32 has optimised INT8 kernels. Unstructured pruning would not reduce latency on a microcontroller without sparse kernels, and knowledge distillation adds a teacher model and training effort that a model of this size does not need; both remain options for future work. The settings used were:

| Setting | Value used |
| --- | --- |
| Method | Post-training full-integer quantisation, TensorFlow Lite converter (Optimise.DEFAULT, INT8 built-in ops only) |
| Weights/activations | INT8; weights per-channel (symmetric), activations per-tensor (asymmetric), the converter defaults |
| Calibration data | 200 windows drawn at random from the training set (all three classes) |
| Input/output tensors | INT8 input and output; input scale 0.0465, zero-point −1 (accelerometer clipped to ±2 g) |
| Target runtime | TensorFlow Lite for Microcontrollers with ESP-NN kernels (planned, not yet deployed) |
| Training | Adam, 25 epochs, batch 64, 10% validation split, seed 42 |

## 3.3 Results and the accuracy–latency–size trade-off

| Metric | FP32 | INT8 | Change |
| --- | --- | --- | --- |
| Model file (flash) | 17.3 KB | 9.9 KB | 1.75× smaller (−43%) |
| Weights only | 12.7 KB | 3.2 KB | 4× smaller |
| Peak activation memory (tensor arena)* | ≈ 11.9 KB | ≈ 3.0 KB | 4× smaller |
| Operations per window | ≈ 208 k MACs | ≈ 208 k MACs | same count, cheaper per MAC |
| Latency on ESP32† | ≈ 13–35 ms | ≈ 4–17 ms | estimate, not measured |
| Accuracy on synthetic test set | 100% | 100% | no difference measurable; not meaningful |

Table 1. FP32 versus INT8, measured by converting the reference model. *Largest pair of live tensors, before runtime overhead. †Assumed 15–40 (FP32) and 5–20 (INT8) cycles per MAC at 240 MHz; must be replaced by a measurement on the board.

A model lies on the Pareto frontier if no other model is better in one objective without being worse in another. INT8 quantisation reduced the weights and the activation memory fourfold, and, by the estimate, shortened inference, without any accuracy difference on the synthetic test set. The whole file shrinks by only 1.75× because a model of 3,187 parameters is dominated by fixed file overhead, a useful reminder that large compression ratios apply to large models. All the INT8 figures meet H1 (flash well below 100 KB, arena about 3 KB, inference below 50 ms), but so does FP32, so the budget is not the deciding factor here. What INT8 buys is memory margin and lower CPU time for a possible larger model. Because the accuracy comparison is on synthetic data, the key claim that INT8 is a Pareto improvement is still unproven: on real data, rare-class (fall) recall is the metric most likely to suffer from quantisation and must be compared with FP32.

# 4. Self-Reflection

## 4.1 Limitations and future work

- No real data and no hardware test (the main limitation). The model was trained and tested on synthetic windows, so the 100% accuracy says nothing about real people; the generator is simple and the classes are easy to separate. Future work: train and test on public datasets (UCI-HAR, MobiAct, SisFall) and on own recordings from several subjects, evaluate with leave-one-subject-out testing, report per-class recall and F1 (falls are rare), then deploy on the ESP32 and measure.

- Estimated latency and energy. Both are calculated from assumptions (Table 1, Section 2.2). Future work: time inference with a hardware timer and measure current with a profiler over a long run.

- Sensor range and noise. At ±2 g the accelerometer saturates during the impact of a fall, which can exceed this value, and the low-cost MPU6050 drifts. Future work: use ±8 g (or ±16 g) and fuse accelerometer and gyroscope with a complementary or Extended Kalman filter.

- Quantisation risk and robustness. INT8 may reduce fall recall on real data; quantisation-aware training is the next step if so. Long-run effects of temperature and I2C noise are untested, and the firmware should check sensor identity and discard stuck or missing samples.

## 4.2 Concept applied: the accuracy–latency–size trade-off (Pareto frontier)

What it means. On an embedded device, accuracy, latency and model size cannot all be improved at once. A design is on the Pareto frontier when no other feasible design is better in one objective without being worse in another, so engineering means choosing a point on that frontier that satisfies the application, not maximising one metric. Why it matters for AIoT. The device has fixed limits on flash, SRAM, compute and battery, so a model that is the most accurate on a laptop may be unusable on the node. Every cost in the model (parameters, operations) becomes a cost in the device (memory, CPU time, energy), and for a battery device energy is effectively a fourth axis. How I applied it. I treated the trade-off as a set of explicit design decisions with thresholds from H1, not as a single optimisation step:

- Model precision: FP32 to INT8 gave 4× smaller weights and activation memory and, by estimate, faster inference, with no accuracy difference on synthetic data (Section 3.3); pruning and distillation were rejected as poorer value for this model size.

- Architecture: a small 1D-CNN was chosen over recurrent or attention models because their memory and compute would not fit the SRAM and 50 ms inference budget.

- Window and sampling: a 128-sample window (1.28 s) balances recognition accuracy, which needs enough context, against fall-alert delay, which grows with window length. The 50% overlap doubles the inference rate in return for fewer missed events, and 100 Hz sampling keeps bus and CPU load low while resolving impacts.

- Energy: the estimate showed the sampling loop and the sensor, not inference, dominate the budget, so the trade-off axis that matters most for battery life is the sensor duty cycle, not model speed.

## 4.3 Evolution of my understanding of AIoT

At the start I treated AIoT as machine learning that happens to run on a small board, where accuracy was the main goal. The project showed me that it is a co-design problem: the sensor range, sampling rate and window length fix the model input; the microcontroller fixes the model size and latency; and the duty cycle fixes the battery life. A change in any one of them moves the others. Two results surprised me. Quantisation compressed the weights fourfold but the model file only 1.75×, because a tiny model is dominated by fixed overhead; and my energy estimate showed that the sensor and sampling loop, not the neural network, dominate the battery budget. I also learned to separate what I measured, what I estimated and what I have not tested, and I now judge an AIoT design by a stated hypothesis and by where it sits on the accuracy–latency–size–energy trade-off, not by accuracy alone.

## 5. Declaration of AI Tools Usage

Tools: Google Gemini 1.5 Advanced: initial structuring and drafting; Claude Sonnet 5.5 (Anthropic): review and restructuring of the report, writing the reference Python script (har_experiment.py) that generates the synthetic data and converts the model, and technical feedback. Purpose: improving structure, clarity and technical wording and producing the reference results. [I ran the script myself and checked the figures in Table 1 against its output.] The project idea, the design decisions and the interpretation of the results are my own.